NI-9481 - Relay Module

Provides switched 24 VDC to pneumatic Z-stage on relay closure 0 and to the warning light on relay closure 1.  Should be renamed “DAQ_Relay” in the NI configuration tool.

NI-9421 - Digital Module

Detects if the Z stage is fully up using a magnetic switch which should provide a DC signal to pin DIO 1.